WNS  16g / 1.6mm ROLLFORMER

Principally used for joining sheet metal at right angles on straight runs, the machine comes with PITTSBURGH LOCK ROLLS.

Auxiliary  shafts are fitted as standard, onto which other rolls such as DOUBLE SEAM ROLLs can be fitted

PITTSBURGH LOCK                        CLOSED PITTSBURGH LOCK

A Flanging Attachment can be fitted for forming right angle flanges on curved, straight and irregular shapes. It is fitted with a tilting head for forming complete circles

PITTSBURGH      DRIVE             DOUBLE             PLAIN            RIGHT            STANDING

Auxiliary Rolls        (STANDARD )      CLEAT*             SEAM*              SEAM*          ANGLE*            SEAM*

| Material Used | 32mm | 54mm | 32mm | 11.0mm | 11.0mm | 54.0mm |
| --- | --- | --- | --- | --- | --- | --- |
| Max. Thickness | 1.6mm | 1.0mm | 1.2mm | 1.0mm | 1.2mm | 1.2mm |
| Seam Width | 13mm | 29mm | 13mm | 10.0mm | - | - |
| Flange Height | - | - | - | - | 11.0mm | - |
| Seam Height | - | - | - | - | - | 19.0mm |

* Optional

SPECIFICATIONS

| LENGTH | WIDTH | HEIGHT | WEIGHT | MOTOR |
| --- | --- | --- | --- | --- |
| 1455mm | 615mm | 1000mm | Approx. 250kg | 2HP (1.5kw) |

All capacities listed are based on the use of Rollforming quality mild steel.

We reserved the right to alter any specifications. All dimensions and weights are approximate