http://www.dipresses.com/newpage1.htm

Quickmaster DI 46-4

Technical specifications

Formats
Max. sheet format: 460 mm x 340 mm (18 1/8" x 13 3/8")
Min. sheet format: 140 mm x 89 mm (5 1/2" x 3 1/2")
Largest printable area: 450 x 330 mm (17 3/4" x 13")
Gripper margin: 10 mm (13/32")
Stock thickness: 0.06 to 0.30 mm (0.0016" to 0.012")

Direct Imaging
Imaging resolutions: 2,540 dpi
Imaging times: < 8 min. at 1,270 dpi
Laser diodes per printing unit: 8

Printing Plates
Presstek PEARL dry, multilayer masters for thermal imaging on supply reel, polyester base material.
No. of masters per reel: app. 35

Blanket cylinders
Length x width, metal edged: 555 mm x 337 mm
(21 7/8" x 13 1/4")
Thickness: 1.95 mm (0.077")
Blanket cylinder undercut: 2.3 mm (0.091")

Inking unit
No. rollers: 8 (incl. fountain roller)
Form rollers: 3
Diameter of form rollers: 54.7 - 45 - 50 mm
(2 1/8" - 1 3/4" - 2")

Pile heights
Feeder: 490 mm (2'7 1/4´´)
Delivery: 450 mm (1'5 3/4")

Connection power
Power consumption (at 50/60 Hz): approx. 10.5kW

About the press
Dimensions (length x width x height):
245 x 255 x 190 cm 
(8' 1/2" x 8' 4 1/2" x 6'3 1/2")
Weight: 3,800 kg (8,378 lbs.)
Emission values: <80 dB (A)

Raster Image Processor

Hardware
High-performance IBM Intel workstation
Software
Delta Technology (optional: Harlequin ScriptWorks)

http://www.merrickind.com/MPC/Heidelberg.HTM

Specifications:

- Direct Imaging of the four printing plates with 64 laser diodes

- Simultaneous plate registration in six minutes

- Sharp-edged screen dot

- Fully automatic print preparation at the push of a button

- Automatic plate changing, cleaning, and blanket wash-up

- Waterless offset printing

- CP Tronic digital press control

- Up to 10,000 sheets/hour production rate

- Standard production sheet size 12" x 18"

- PostScript Level 2 RIP for Macintosh and PC compatibility

- Harlequin Interpreter

- Digital AlphaStation 200 4/233

- 64 Bit-RISC architecture

- 64 MB RAM, 233 MHz cycled

- Operating System - Windows NT Server 4.0

- Network interface - Ethernet with TCP/IP and Appletalk

- Soft proof check of bitmap data in various image sizes on the server and on the printing press
___________________________________________________________