TIIGER.. .. .. Series

TECHNICAL FEATURES

TIGER series floodlights consist of an aluminium body fitted with a lampholder and a tempered glass cover fastened with a hinge and a hermetic seal with aluminium clips.

CONFORMITY TO STANDARDS

Explosion-proof floodlights built to CENELEC standards EN 60079-15:2005, EN 60079-0: 2006, EN 61241-1: 2004, EN 61241-0:2006 and EUROPEAN DIRECTIVE 94/9/EC: 1994.

INSTALLATION AREAS

These floodlights can be installed indoors or outdoors in potentially hazardous atmospheres containing explosive or combustible gases and dusts. They are installed in the following zones: zone 2, zone 22.

APPLICATIONS

Potentially explosive atmospheres

- - chemical, petrochemical and pharmaceutical industries;

- - onshore and offshore ship industries;

- - areas with risks of explosion and fire;

- - OIL and GAS industries

- CERTIFICATION AND USE

- Execution CE 0722:  II 3GD Ex nR II T2/T3 Ex tD A22 IP66 (Protection against combustible gases and dusts)

- II 3D Ex tD A22 IP66 (Protection against combustible dusts)

- Ambient temperature --->  standard: -25°C +45°C

- --->  other range admitted: -50°C +45°C;  -50°C +60°C

- CE test certification: CESI 06 ATEX 054

- TECHNICAL SPECIFICATIONS

- Lampholder: EDISON 40 (E-40)

- Rated voltage: 110÷240 V 250V 277V

- Rated frequency: 50÷60 Hz

- Wiring: Direct connection to the terminal box  L, N, Pe section 4 mm2.

- Power factor: 0.90

- STRUCTURE

- • Aluminium die-cast body (EN AB 46100)

- • Tempered glass cover

- • PG13 polyamide 6.6 cable gland: tug-proof and self-estinguishing Vo (9 ÷ 11 mm cable)

- • Wiring consisting of silicone rubber cables with glass braid

- • Silicone seals

- • Symmetric or asymmetric reflectors in oxidized and polished aluminium plate 99.85

- • RAL7035 grey polyester powder coating